Table 2. Robustness Checks — Panel Regression

Dependent variable: average hourly demand (MWh). All specifications include BA and month-year fixed effects, HDD, CDD controls (except S7), and wild cluster bootstrap inference (Webb weights, 999 reps.).

| Specification | N | Coefficient | 95% CI | p-value |
| --- | --- | --- | --- | --- |
| Panel A: Lag Sensitivity |  |  |  |  |
| S1: 18-month lag (primary) | 198 | 0.029 | [−0.125, 0.132] | 0.063 |
| S2: 12-month lag | 216 | 0.047 | [−0.583, 0.667] | 0.128 |
| S3: 24-month lag | 180 | 0.064 | [−0.495, 0.542] | 0.221 |
| S4: 30-month lag | 162 | −0.100 | [−0.414, 0.292] | 0.100 |
| Panel B: MW Threshold Sensitivity |  |  |  |  |
| S5: ≥50 MW threshold | 198 | 0.025 | [−0.062, 0.078] | 0.064 |
| S1: ≥100 MW threshold (primary) | 198 | 0.029 | [−0.125, 0.132] | 0.063 |
| S6: ≥200 MW threshold | 198 | 0.001 | [−0.417, 0.390] | 0.905 |
| Panel C: Control Specification |  |  |  |  |
| S1: Baseline — HDD, CDD, GDP (primary) | 198 | 0.029 | [−0.125, 0.132] | 0.063 |
| S7: No GDP control | 198 | 0.013 | [−0.206, 0.250] | 0.099 |
| S8: BA-specific linear trends | 198 | −0.059 | [−1.670, 1.917]† | 0.783 |
| S9: BA-specific quadratic trends | 198 | −0.039 | [−1.250, 1.725] | 0.840 |
| Panel D: Queue Variable |  |  |  |  |
| S1: Large projects only, ≥100 MW (primary) | 198 | 0.029 | [−0.125, 0.132] | 0.063 |
| S10: All projects, no threshold | 198 | 0.022 | [−0.054, 0.149] | 0.098 |

Notes: All regressions estimated via reghdfe with BA and month-year fixed effects. BA fixed effects absorbed by including i.ba_num explicitly as regressors to maintain boottest compatibility (see text). Queue regressor winsorized at 95th percentile (18,588 MW) in all specifications; two MISO observations at 154,214 MW and 102,870 MW reflect a documented 2022 queue reform batch-filing artifact lagged 18 months forward. Primary specification (bold) is S1: 18-month lag, ≥100 MW threshold, full controls. Coefficients interpreted as MWh change in monthly average hourly demand per additional MW of qualifying queue filings 18 months prior. Panel A: coefficient rises from 12 to 24 months then turns negative at 30 months, consistent with the construction timeline distribution for hyperscale facilities. Panel B: 50 MW and 100 MW thresholds produce nearly identical results; 200 MW threshold collapses due to insufficient within-BA variation. Panel C: attenuation under BA-specific trends (S8, S9) is expected—these trends absorb secular ERCOT growth that may itself be data-center-driven. †S8 confidence set is disconnected [−1.670, −1.542] ∪ [−1.417, 1.917]; outer bounds reported. Panel D: all-projects coefficient (S10) slightly below large-only (S1), indicating the demand signal is concentrated in but not exclusive to ≥100 MW projects.