This document describes the PJM interconnection queue data. The base data set is pjm_main_data_for_public.csv.  This base data set contains two types of variables. The first type are variables collected directly from the engineering studies.  The second type are geographic variables that other researchers would not be able to reconstruct themselves, variables based on the geographic coordinates of PJM interconnection requests and substations.

The second data set is pjm_main_data_for_public_with_additional_vars.csv.  It is constructed by running code_pjm_for_public.Rmd on the base data set. This code makes imputations most researchers will want to perform, and it details the logic and assumptions for these imputations.  It also merges the base data with additional data on these interconnection requests from PJM: PlanningQueues_2024.xlsx, which we downloaded from PJM’s website.

For each variable, this document provides a description and, where applicable, an overview of the data collection protocol we used. The accompanying data dictionary, pjm_main_data_dictionary_for_public.csv, includes definitions, labels, and data types for the variables in pjm_main_data_for_public.csv.

These data cover all requests for interconnection that PJM received from 2008 to 2020. Data are collected from the engineering studies available for these requests as of August 2024.  Several of the later requests are still active in the queue and waiting for additional studies. Observations are unique at the level of the queue number and study.

Interconnection Requests:

- Queue Number: Each generator is assigned a unique queue number when it enters the interconnection queue. We record this number in the variable queue_number.

- Versions: The engineering studies issued to generators are often revised. We collect data from the most recent version available for each study. We record the version number of the study we collected in the variable version. If a study indicates that it was revised but does not specify the version number, we record the version as “R”.

- Study: As part of the interconnection process, PJM requires most generator requests to undergo three engineering studies: a Feasibility Study, a System Impact Study, and a Facilities Study. We record the study name in the variable study. In the R code, we classify these studies as Study 1, Study 2, and Study 3, respectively, in the variable study_id, PJM may issue combined studies as part of an expedited interconnection process, in which case we record the study as both names.  For example, if the observation is a combined Feasibility and Impact Study, we record the study variable as “FeaImp”.

- Last Study: We record the last study issued to the interconnection request as of August 2024 in the variable last_study and its numeric classification in the variable last_study_id.

- Developer: The engineering studies often list the developer associated with the request in the System Impact and Facility studies. We record this company in the developer variable; if the developer is an LLC, we attempt to match it to its parent company and record that instead. We also infer developers based on news articles and other sources when possible, so the information in this variable does not only come from the engineering studies.

Request Characteristics:

- Uprates: Interconnection requests may include proposals to increase the energy or capacity resource of an existing generator that has already been completed or is already waiting in the interconnection queue. The uprate variable is coded as 1 if the request is an uprate and 0 if it is not. When the study explicitly mentions the queue number of the original generator that the request is an uprate to, we record that number in the variable uprate_to.

- Fuel Type: We obtain the fuel type variable fuel from the publicly available PJM planning queue data. In R, we extract the first listed fuel and record it as the primary fuel type in the variable fuel_prim. We also generate the variable wind_solar which is an indicator for if the fuel type includes wind or solar.

- Status: We obtain the status of each request from the publicly available PJM planning queue data and classify them in the R code. Interconnection requests are classified as completed and assigned a status_code variable of 1 if their status is “In Service” or “Partially in Service – Under Construction.” Requests are classified as active and assigned a status_code of 2 if their status is “Engineering and Procurement,” “Under Construction,” “Active,” or “Suspended.” Requests are classified as withdrawn and assigned a status_code of 3 if their status is “Deactivated,” “Canceled,” or “Withdrawn.”

- Geography: We obtain the state variable state and the county variable county from the PJM planning queue data. When these variables are missing, we impute them based on the request’s geographic coordinates.

- Transmission Owner: We obtain the transmission owner variable transmission_owner from the PJM planning queue data. When this variable is missing, we impute it based on the request's geographic coordinates.

Dates and Wait Time:

- Queue Date/In Service Date/Withdrawal Date: We obtain a series of date variables from the publicly available PJM planning queue data and convert them into year-month format in R. queue_date is the date the interconnection request was made. in_service_date indicates when the generator corresponding to the request began commercial operation, and withdrawal_date is the date when the generator corresponding to the request withdrew from the interconnection process.

- Out Queue Date: We construct a variable out_queue_date in R to track when a request leaves the interconnection queue. For completed requests, out_queue_date is set to the in_service_date. For withdrawn requests, it is set to the withdrawal_date. For active requests, the out_queue_date reflects the projected in-service date.

- Study Issue Date: We collect the date when the engineering study was issued and record it in variable issue_date, in year-month format. While very few observations for the second and third studies are missing issue dates (0.3%), Study 1 issue dates are often missing: 14% are missing completely and 0.5% only have the year. The R code imputes issue dates for these observations by assuming they are 4 months after the queue_date, in accordance with PJM’s official timeline.

- Wait Time: We construct a series of wait time variables study1_wait, study2_wait, and study3_wait in the R code. The wait time for the Study 1 is defined as the number of months between the queue_date and the Study 1 issue_date. The wait time for later studies is the time between their issue dates.

Size:

- Request Capacity: Each interconnection request specifies the requested megawatts (MW) of energy resource and the requested MW of capacity resource. We collect these values from the engineering studies and record them in the variables energy_mw and capacity_mw. The studies sometimes explicitly provide the requested maximum facility output; in these cases, we record this information in the variable mfo.

- Size: We construct the variable size_mw in the R code. This variable is the maximum of the generator's requested energy and requested capacity in MW.

Costs:

The engineering studies report cost estimates for several categories of cost. If a study explicitly reports a zero-cost estimate or states that no such costs were identified, we record the cost as zero. If no cost estimate is provided, the cost variable is set to missing. We record all cost variables in nominal terms as they appear in the study. We collect the version excluding taxes when both are reported. The R code converts all cost variables to real 2020 dollars

- POI cost: The POI cost, also known as the physical connection cost, is the estimated cost to connect a generator to the electric grid. In the engineering studies, this cost is typically broken down into three components: direct connection cost, indirect connection cost, and attachment cost. We calculate the total POI cost as the sum of these three components and record it in the variable poi_cost. The individual components are recorded in the variables direct_cost, nondirect_cost, and attachment_cost.

- Combined Cost: When multiple requests contribute to overloading the transmission grid, PJM shares the cost of the system upgrades necessary to resolve the overload across these requests, in proportion to their contribution to the overload. The combined cost is the total estimated cost for all potential network upgrades a generator may share responsibility for. We record this value in the variable combined_cost.

- Network Upgrade Cost: The network upgrade cost is the estimated cost for transmission network upgrades that a generator is responsible for paying. We extract these costs from sections of the engineering studies that refer to network upgrade costs, specifically the new system upgrade cost and the previously identified system upgrade cost. The portion of the shared network upgrade costs that the generator is responsible for are not usually reported until Study 2, so this variable is almost always missing for Study 1. We record the total network upgrade cost in the variable network_upgrade_cost, and its two components in the variables  newsystem_upgrade_cost and prevsystem_upgrade_cost.

- Total Interconnection Cost: The total interconnection cost is the sum of the POI cost and the network upgrade cost. We record it in the variable total_cost. Costs that are not clearly allocated to the request are excluded from this total. For instance, in Study 1, network upgrade costs are usually not allocated to individual requests. Such unallocated network upgrade costs are recorded only in the variable combined_cost and are not included in the variable total_cost.

- Per-MW Cost: We construct the per-MW cost variables in R. The variable poi_cost_mw represents the POI cost on a per-MW basis, calculated by dividing the poi_cost variable by the size_mw variable. The same approach is applied to compute the network_upgrade_cost_mw variable and the total_cost_mw variable.

- Network Allocation: We construct the variable network_allocation in R to indicate whether a request received an explicit network upgrade cost in Study 2. This variable is coded as 1 if the generator has a non-missing network upgrade cost estimate and 0 if it does not.

Cost Sharing:

- Cost-share group: PJM may provide information on the cost-sharing of network upgrades by listing the queue numbers of other generators with which a generator shares these costs. We extract this information primarily from Study 2, where it most frequently appears, and apply it to all observations associated with the same generator. These cost-sharing generators are recorded in the variable cost_share_group.

- Study 1 Cost-share: We construct a variable study1_cost_share in R to indicate whether a generator received a non-zero combined cost in Study 1. Receiving a non-zero combined cost suggests that the generator might be responsible for network upgrade costs in Study 2 and Study 3. This variable is coded as 1 if the generator received a non-zero combined cost estimate in Study 1 and 0 if it is not.

Point of Interconnection (POI):

Project developers usually select a substation as their point of interconnection. It is also common for the POI to be a line tap, in which case we classify the substations associated with that line segment as the POI. If the POI is instead a circuit, circuit breaker, transformer, bus, TSS, etc., we record the POI as the substation associated with that electrical infrastructure.

- TapLine: The variable tapline indicates whether a generator requested to connect via a line. It is 1 if the generator is tapping into a line and 0 otherwise.

- Substation/Line: We collect the POI information from both the Point of Interconnection section of the engineering study and the title page of the study. If there were inconsistencies between these two sources (e.g., discrepancies in voltage units), we referred to the Network Impact section of the study for verification. We record this information in the variables substation and line. If a study lists multiple POIs, we collect the primary one; if the report does not specify a primary POI, we collect the one that is listed first.

- Matched POI Substations: The POI names in the studies can vary from PJM’s official substation names, so we match them to a standardized list of PJM substations. The matched POI is recorded in the variable poi_substation, and up to four official substation names associated with the POI are recorded in the variables matched_substation1 through matched_substation4, with their corresponding unique ID variables substation_id1 through substation_id4. We also calculate the distance from the generator to each matched POI substation, in km, based on their geographic coordinates. We record the distance in the variables dist_matched_substation1 through dist_matched_substation4. The ID of the nearest matched substation is stored in nearest_matched_substation_id, and its distance in min_dist_to_matched_substation.

- Distance to POI: Because the POI is not always a substation, we also create variables with the distance from the generator to each POI in km and record it in the variables dist_poi1 to dist_poi4. For substations, the distance is calculated as point-to-point. For lines, it is the perpendicular distance to a direct line between the two substations. The shortest of these distances is stored in the variable min_dist_to_poi.

- Nearest Substation: The requested POI is not always the nearest substation. We identify and record the nearest substation to each request—regardless of the POI—in the variable nearest_substation and its distance from the request in km in the variable dist_nearest_substation.

- Voltage: We extracted the voltage of the POI that the generator is requesting to connect to and recorded it in the variable voltage.

- Coordinates: The variables lon_queue_noise and lat_queue_noise are the longitude and latitude of the interconnection request. We added randomized noise uniformly drawn from -0.01 to 0.01 degrees to the original coordinates.

Engineering Tests:

PJM conducts multiple engineering tests to identify whether the request triggers any issues and, if so, what remediation is required. As a general rule: if a problem is identified, we record "Yes"; if the test is performed and no issue is found, we record "None"; and if the test is not performed, we record the value as "NA". Typically, the engineering studies are conducted under two scenarios: summer peak and light load. When this occurs, we record the results from the summer peak analysis.

- Generator Deliverability: This test evaluates whether the generator can deliver power reliably under normal system conditions. We record the test result in the variable generator_deliverability.

- Multiple Facility Contingency: This test examines the system's ability to withstand multiple simultaneous equipment failures. We record the test result in the variable multiple_facility_contingency.

- Contribution to Previously Identified Overload: This test assesses whether the generator contributes to an overload that has already been identified in previous studies. We record the test result in the variable contrib_to_prev_identifi.

- New System Reinforcement: This test identifies whether the generator creates new system violations that require upgrades. These are also referred to as new system upgrades in the study. We record the test result in the variable newsystem_reinforcement.

- Previously Identified System Reinforcement: This test determines whether the generator contributes to previously known system issues that still require reinforcement. This test also referred to as previously identified system upgrades in the study. We record the test result in the variable prevsystem_reinforcement.

- Short Circuit Analysis: This test determines whether the generator would cause short circuit current levels to exceed equipment ratings. We record the test result in the variable short_circuit.

- System Protection Analysis: This test examines whether the existing protection can properly isolate faults involving the new generator. We record the test result in the variable system_protection_analysis.

- Short Circuit and Dynamic Analysis: This test evaluates how the generator may impact system stability and dynamic response under fault conditions. This test is also referred to as dynamic simulation analysis, stability and reactive power requirement, stability analysis, and transient stability analysis in the study. We record the test result in the variable short_circuit_dynamic_analysis. We apply a slightly different rule for this variable: if a problem is identified and mitigation by the generator is required, we record "Yes"; if no problem is identified or a problem is identified but mitigation by the generator is not required, we record "None"; and if the test is not performed, we record the value as missing.

New Infrastructure:

The engineering studies may identify new electrical infrastructure, such as substations or circuit breakers, that the generator is required to construct. As a general rule: we record “Yes” if new infrastructure needs to be constructed, and “No” if does not. However, exceptions apply: In a Feasibility or Impact Study, if the system reinforcement section indicates the need to build a new substation or circuit (with shared costs), but this requirement is not mentioned in the attachment facility section, the direct connection section, or the indirect connection section, the case is classified as "No". We classify it as no because the generator may not ultimately be responsible for the cost of the infrastructure. In contrast, in a Facility Study, if the generator is directly responsible for constructing a new substation or circuit, regardless of whether it is for system reinforcement, the case is classified as "Yes."

- New Substation: We record the variable new_substation as "Yes" if the interconnection request requires constructing any type of new substation, including substations, switching stations, breaker stations, ring bus stations, or switchyards.

- New Circuit Breaker: We record the variable new_circuit_breaker as “Yes” if the request requires constructing a new circuit breaker.

Previously Completed Interconnection Requests:

We construct a set of variables that capture information about the most recently completed, nearby request. For each request, we identify the latest interconnection request that began operation before the issue date of the current study, within a specified geographic region. From that nearby generator, we extract and record both the total interconnection cost on a per-MW basis total_cost_mw and the request size in MW size_mw. We define "nearby" using five geographic levels: (1) at the same substation, (2) within a 10 km radius, (3) within a 20 km radius, (4) within a 50 km radius, and (5) at the same substation and within 10 km. We calculate the distance using the coordinates for the requests. We record these values in the variables prev_cost_sub through prev_size_sub10km. All cost values are adjusted for inflation using the Consumer Price Index (CPI) to reflect real 2020 dollars, with the adjustment based on the year the study was issued

Queue Size:

We construct a set of variables in R that indicate the size of the interconnection queue, measured by the number of generators waiting and the total capacity of waiting generators in MW.

- Queue Size at entry: We calculate the size of the interconnection queue at the year-month of the date the request for interconnection was submitted. The number of generators is recorded in the variable queue_size_at_queue, and the total capacity in MW is recorded in the variable queue_sizeMW_at_queue. We also disaggregate the size of the interconnection queue by fuel type and record these values in the corresponding variables.

- Queue Size at first study issuance: We calculate the size of the interconnection queue at the year-month when the generator receives its Study 1. The number of generators is recorded in the variable queue_size_at_study1, and the total capacity in MW is recorded in the variable queue_sizeMW_at_study1.

- Queue Size at second study issuance: We calculate the size of the interconnection queue at the year-month when the generator receives its study 2. The number of projects is recorded in the variable queue_size_at_study2, and the total capacity in MW is recorded in the variable queue_sizeMW_at_study2.